# WiMAX

WiMAX (Worldwide Interoperability for Microwave Access) is a communication technology for wirelessly delivering high-speed Internet service to large geographical areas. The 2005 WiMAX revision provided bit rates up to 40 Mbit/s[1][2] with the 2011 update of up to 1 Gbit/s for fixed stations. It is a part of a “fourth generation,” or 4G, of wireless-communication technology. WiMax far surpasses the 30-metre (100-foot) wireless range of a conventional Wi-Fi local area network (LAN), offering a metropolitan area network with a signal radius of about 50 km (30 miles). The name "WiMAX" was created by the WiMAX Forum, which was formed in June 2001 to promote conformity and interoperability of the standard. The forum describes WiMAX as "a standards-based technology enabling the delivery of last mile wireless broadband access as an alternative to cable and DSL".[3] WiMax offers data-transfer rates of up to 75 Mbit/s, which is superior to conventional cable-modem and DSL connections. However, the bandwidth must be split among multiple users and thus yields lower speeds in practice.[4]

## Terminology

WiMAX refers to interoperable implementations of the IEEE 802.16 family of wireless-networks standards ratified by the WiMAX Forum. Similarly, Wi-Fi, refers to interoperable implementations of the IEEE 802.11Wireless LAN standards certified by the Wi-Fi Alliance. WiMAX Forum certification allows vendors to sell fixed or mobile products as WiMAX certified, thus ensuring a level of interoperability with other certified products, as long as they fit the same profile.

The original IEEE 802.16 standard (now called "Fixed WiMAX") was published in 2001. WiMAX adopted some of its technology from WiBro, a service marketed in Korea.[5]

Mobile WiMAX (originally based on 802.16e-2005) is the revision that was deployed in many countries, and basis of future revisions such as 802.16m-2011.

WiMAX is sometimes referred to as "Wi-Fi on steroids"[6] and can be used for a number of applications including broadband connections, cellular backhaul, hotspots, etc. It is similar to Wi-Fi but it can also permit usage at much greater distances.[7]

## Uses

The bandwidth and range of WiMAX make it suitable for the following potential applications:

- Providing portable mobile broadband connectivity across cities and countries through a variety of devices.

- Providing a wireless alternative to cable and digital subscriber line (DSL) for "last mile" broadband access.

- Providing data, telecommunications (VoIP) and IPTV services (triple play).

- Providing a source of Internet connectivity as part of a business continuity plan.

- Smart grids and metering

### Internet access

WiMAX can provide at-home or mobile Internet access across whole cities or countries. In many cases this has resulted in competition in markets which typically only had access through an existing incumbent DSL (or similar) operator.

Additionally, given the relatively low costs associated with the deployment of a WiMAX network (in comparison with 3G, HSDPA, xDSL, HFC or FTTx), it is now economically viable to provide last-mile broadband Internet access in remote locations.